[ DRAFT — WORK IN PROGRESS ]

This document is a preliminary draft. Content, figures, and results
are subject to revision. Final performance results pending board testing.

Graduation Project Report

Design and Implementation of a ShuffleNet Accelerator
on FPGA with RTL and HLS

iWave ZU19EG Development Kit  |  Zynq UltraScale+ MPSoC

Team Members

Ahmed Ahmed Nageeb Ahmed Elbermawy

Yousef Abdulrahman Abdulnabi Abdulrahman

Mohamed Ahmed Roshdy Saad

Basmala Hatem Abdullah Mostafa

2026

# Draft Notice

This document is a DRAFT version of the graduation project report that contains only the main points. It is intended to provide an overview of the design, implementation, and preliminary results for review and feedback purposes.

The following sections are partially complete:

Abstract and Introduction

Background and system architecture description

RTL implementation details for all three processing groups

Verification results (48 testbenches, all passing)

HLS model descriptions

Design optimization summary

Post-implementation resource utilization and timing results

The following sections require board-level testing to be finalized:

Board inference latency and throughput measurements

On-chip power consumption

RTL vs. HLS performance comparison table

Final accuracy measurements on the physical FPGA

Figures in this report include the system block diagram and accelerator dataflow diagram generated from the repository. Additional waveforms and timing diagrams will be added in the final version.

# Abstract

This report presents the design and implementation of a real-time image classification accelerator based on the ShuffleNet V2 × 1.0 convolutional neural network, targeting the iWave ZU19EG Development Kit (Xilinx Zynq UltraScale+ MPSoC, part xczu19eg-ffvc1760-1-i). The work is organized into two parallel implementation tracks that share the same target silicon, enabling a direct and fair comparison.

The first track is a hand-crafted RTL Verilog accelerator that achieves timing closure at 100 MHz using a Q6.8 fixed-point datapath. All convolution weights are quantized and stored in on-chip Block RAMs (BRAMs), eliminating the need for external DDR memory access during inference. The ARM Cortex-A53 Processing System (PS) loads images and reads results through a 32 MB AXI4-Lite interface. Post-implementation results show 1,550 DSP48E2 slices (78.76%), 550.5 BRAM tiles (55.95%), and 68,299 LUTs (13.07%) on the ZU19EG, with a worst-case timing slack of +0.125 ns confirming full timing closure.

The second track provides four progressively optimized Vitis HLS C++ reference models (Baseline, Pipelined, Parallelism, Final) targeting the same device. These models demonstrate the HLS optimization design space and serve as a benchmark for the RTL efficiency advantage.

All 48 synthesizable RTL modules are verified by self-contained, self-checking Verilog testbenches that pass without failures under Vivado XSim 2024.2. A bit-accurate Q6.8 Python simulation model achieves approximately 71% Top-1 accuracy on 100 ImageNet validation images, confirming the numerical correctness of the BN-folded fixed-point quantization scheme. Board-level performance results are pending hardware deployment.

# 1. Introduction

The rapid adoption of convolutional neural networks (CNNs) in computer vision applications — from autonomous driving and medical imaging to consumer electronics — has created a pressing need for hardware accelerators that can execute inference efficiently at the network edge. Unlike cloud-based inference, which benefits from dedicated GPU clusters and reliable high-bandwidth connectivity, edge inference must satisfy strict constraints on latency (typically < 50 ms for responsive applications), power consumption (milliwatts to a few watts), cost, and offline operation capability.

Conventional embedded processors, such as the ARM Cortex-A53 incorporated in the Zynq UltraScale+ SoC, cannot meet real-time latency targets for modern CNNs without dedicated hardware acceleration. Running a ShuffleNet V2 inference pass in software requires approximately 146 million multiply-accumulate (MAC) operations. Even with NEON SIMD vectorization, the Cortex-A53 achieves at most 4 GFLOPS, resulting in roughly 36 ms per frame — barely meeting 30 fps but consuming the full CPU bandwidth.

Field-Programmable Gate Arrays (FPGAs) are uniquely suited to CNN acceleration at the edge. Their reconfigurable fabric enables massively parallel fixed-function arithmetic datapaths custom-tailored to the structure of a specific network, with deterministic execution latency, low power at moderate throughput, and the ability to exploit reduced-precision arithmetic through custom datapath design. The Xilinx Zynq UltraScale+ MPSoC combines a powerful ARM processing system with a large programmable logic fabric and high-bandwidth AXI interconnect on a single chip, making it an ideal platform for tightly coupled PS+PL inference systems.

This project implements ShuffleNet V2 × 1.0 — a state-of-the-art mobile CNN designed specifically for efficient edge inference — on the iWave ZU19EG Development Kit. Two complementary implementation tracks are provided:

RTL Track: A fully hand-crafted Verilog RTL implementation with a Q6.8 fixed-point datapath, three-group pipelined architecture, and all weights stored on-chip. This represents the primary hardware contribution of the project.

HLS Track: Four progressively optimized Vitis HLS C++ models targeting the identical ZU19EG silicon, providing a direct, same-chip comparison of RTL efficiency versus automated high-level synthesis at each optimization level.

## 1.1 Contributions

The specific technical contributions of this project are:

A complete, timing-closed RTL implementation of ShuffleNet V2 × 1.0 that achieves 100 MHz operation on the Zynq UltraScale+ ZU19EG without requiring external memory access during inference.

A per-layer fixed-point width optimization framework that reduces datapath widths from a uniform 15-bit baseline to 8, 10, 12, and 9 bits per layer, minimizing resource usage while preserving ~71% Top-1 accuracy.

A novel split out-of-context (OOC) synthesis methodology that reduces peak workstation memory requirements from 34 GB to under 16 GB, enabling synthesis on standard development machines without specialized hardware.

48 self-contained, self-checking Verilog testbenches with inline golden model functions, covering every synthesizable RTL module. All 48 pass without failures.

A bit-accurate Q6.8 Python simulation model that enables offline verification of BN-folded fixed-point weights before hardware synthesis.

Four Vitis HLS C++ reference models targeting the same ZU19EG silicon for a quantitative RTL vs. HLS comparison of performance, resource usage, and design effort.

## 1.2 Report Organization

The remainder of this report is structured as follows: Section 2 provides background on the ShuffleNet V2 architecture and fixed-point quantization principles. Section 3 describes the overall system architecture, including the PS-PL interface and memory organization. Section 4 presents the detailed RTL implementation of each processing group, the common arithmetic primitives, the weight storage scheme, and the split synthesis methodology. Section 5 covers the verification approach and results. Section 6 describes the four HLS reference models. Section 7 summarizes the applied design optimizations. Section 8 presents the post-implementation results. Section 9 concludes with a summary and directions for future work.

# 2. Background and Related Work

## 2.1 ShuffleNet V2 Network Architecture

ShuffleNet V2 × 1.0, introduced by Ma et al. in 2018, is a lightweight CNN architecture designed to minimize memory access cost while maximizing computation-to-memory-access ratio. Unlike earlier mobile architectures that optimize purely for floating-point operation (FLOP) count, ShuffleNet V2 specifically targets memory bandwidth efficiency — the true bottleneck on resource-constrained hardware.

The central building block is the ShuffleNet V2 unit, shown conceptually below. The input channels are split into two equal halves. One half passes unchanged through an identity branch. The other half passes through a feature-extraction branch consisting of a 1×1 pointwise convolution (PW), a 3×3 depthwise convolution (DW), and another 1×1 pointwise convolution. The two branch outputs are concatenated and their channels are shuffled to enable cross-channel information mixing. This design achieves strong accuracy at low FLOPs because the identity branch avoids redundant feature computation.

For stride-2 blocks (used at stage transitions to halve spatial resolution), both branches apply convolutions, and the outputs are concatenated (doubling the channel count) rather than summed.

The full network processes 224×224×3 RGB images through an initial convolution and max pool, followed by three stages of ShuffleNet V2 blocks (4, 8, and 4 blocks respectively), a final 1×1 convolution, global average pooling, and a 1000-class fully connected classifier. For the ×1.0 variant, the channel widths progress as 24 → 116 → 232 → 464 → 1024.

| Stage | Layer | Input Size | Output Size | Blocks | HW Group |
| --- | --- | --- | --- | --- | --- |
| — | Conv1 (3×3, stride 2) | 224×224×3 | 112×112×24 | 1 | Group 1 |
| — | MaxPool (3×3, stride 2) | 112×112×24 | 56×56×24 | 1 | Group 1 |
| 2 | Shuffle Blocks (stride-2 + stride-1) | 56×56×24 | 28×28×116 | 4 | Group 2 |
| 3 | Shuffle Blocks (stride-2 + stride-1) | 28×28×116 | 14×14×232 | 8 | Group 2 |
| 4 | Shuffle Blocks (stride-2 + stride-1) | 14×14×232 | 7×7×464 | 4 | Group 2 |
| — | Conv5 (1×1) | 7×7×464 | 7×7×1024 | 1 | Group 3 |
| — | GlobalAvgPool | 7×7×1024 | 1024 | 1 | Group 3 |
| — | FC (1000) | 1024 | 1000 | 1 | Group 3 |

## 2.2 Fixed-Point Quantization and BN Folding

Deploying CNNs on FPGA fabric requires replacing 32-bit floating-point (FP32) arithmetic with integer fixed-point representation. Fixed-point arithmetic replaces expensive floating-point multiply-add units (requiring ~200 LUTs in FPGA fabric) with compact DSP48E2 primitives that execute 18×27-bit signed multiplications in a single clock cycle.

This project uses the Q6.8 format as the baseline: a 15-bit signed integer where the binary point is positioned 8 bits from the LSB (6 integer bits, 8 fractional bits, 1 sign bit). A value V in Q6.8 represents the real number V / 256. The representable range is approximately −128 to +127.996. Per-layer width optimization then narrows this further based on the dynamic range of each layer's activations.

Batch normalization (BN) is a training-time technique that normalizes layer activations to have zero mean and unit variance, then applies a learned scale and shift. At inference, BN parameters are fixed, so they can be absorbed directly into the preceding convolution weights offline:

W_folded[f][c][h][w] = γ[f] × W[f][c][h][w] / √(σ²[f] + ε)

b_folded[f] = β[f] − γ[f] × μ[f] / √(σ²[f] + ε)

where γ, β are BN scale and shift, μ, σ² are the running mean and variance, and f indexes the output filter. This eliminates all run-time BN operations, replacing them with pre-computed folded weights stored in on-chip ROMs. The folding is performed offline in Python using the torchvision pretrained ShuffleNet V2 × 1.0 checkpoint.

## 2.3 FPGA-Based CNN Acceleration

FPGA acceleration of CNNs is an active research area. Key prior works include the Xilinx xDNN inference engine, the Angel-Eye CNN accelerator (Guo et al., 2017), and ZynqNet (Gschwend, 2016). These works demonstrate that custom fixed-point datapaths on FPGAs can achieve competitive throughput and energy efficiency compared to embedded GPUs, particularly for smaller networks where memory bandwidth is not the primary bottleneck.

The distinguishing characteristic of this project is the fully hand-crafted RTL approach (as opposed to HLS-generated or vendor-IP-based implementations), which enables precise control of the datapath at every stage. This contrasts with the parallel HLS track, which demonstrates the same network implemented at a higher abstraction level — providing a concrete, same-silicon comparison of design effort versus efficiency.

# 3. System Architecture

The complete system is organized as a three-level hierarchy: the board-level wrapper (shufflenet_board_top), the accelerator integration layer (accelerator_top), and the three processing groups (group1_top, group2_top, group3_top). Figure 1 shows the system-level block diagram.

Figure 1: System block diagram showing PS → AXI4-Lite → PL accelerator hierarchy.

## 3.1 Processing System and Clock Domain

The Zynq UltraScale+ PS is configured with the iWave ZU19EG board preset, which sets up the DDR4 memory interface, MIO pinout, and PL clock outputs. The PL clock (pl_clk0) is generated at 100 MHz from the PS internal PLL and driven into the PL fabric.

Inside shufflenet_board_top, an MMCM (Mixed-Mode Clock Manager, MMCME4_BASE primitive) re-generates a phase-clean 100 MHz clock for all PL logic. The MMCM uses a VCO frequency of 1000 MHz (CLKFBOUT_MULT_F=10, CLKOUT0_DIVIDE_F=10). Using the MMCM rather than directly using pl_clk0 prevents any PS-side clock jitter from propagating into the accelerator timing paths, improving timing closure margin.

A 4-stage synchronizer chain converts the asynchronous active-low pl_resetn0 from the PS into a synchronous reset for the MMCM clock domain. The MMCM-locked signal is ANDed with the synchronized reset before distribution, ensuring that no logic operates until the MMCM achieves lock.

## 3.2 AXI4-Lite Interface and Address Map

The PS communicates with the accelerator through the M_AXI_HPM0_FPD port (High-Performance Master 0, Full Power Domain), which is a 32-bit AXI4 master at 100 MHz. An AXI SmartConnect IP routes this to the 32-bit AXI4-Lite slave (axi_photo_mem_slave.v) mapped at base address 0xA000_0000 with a 32 MB window.

The slave partitions its address space using bit 21 of the write address (AWADDR):

| AWADDR[21] | Region | Access | Description |
| --- | --- | --- | --- |
| 0 | Pixel write region | Write only | AWADDR[20]=bank, [19:18]=channel, [17:2]=pixel index; WDATA[14:0]=Q6.8 pixel |
| 1 | Control/Status Register (CSR) | Read/Write | WDATA[0]=photo_ready; RDATA[0]=photo_ready, [1]=busy, [2]=done, [12:3]=class_idx |

The PS inference sequence is: (1) write all 150,528 pixels (224×224×3), (2) write CSR[0]=1 to trigger inference, (3) poll CSR until busy=0 and done=1, (4) read class_idx from CSR[12:3], (5) write CSR[0]=0 before the next image.

## 3.3 Accelerator Integration and Pipelining

The accelerator_top module ties together the three processing groups, the extra_mem feature map buffer, and the accelerator_ctrl FSM. The FSM implements a 5-state pipelined execution model that overlaps the execution of different images across the three groups:

| FSM State | Group 1 | Group 2 | Group 3 | Description |
| --- | --- | --- | --- | --- |
| IDLE | Off | Off | Off | Waiting for photo_ready |
| G1 | Image N | Off | Off | G1 processes first image |
| G1+G2 | Image N+1 | Image N | Off | G1 and G2 overlap |
| G1+G2+G3 | Image N+2 | Image N+1 | Image N | Full pipeline active |
| G1+G3 | Image N+3 | Off | Image N+1 | G2 done; G3 classifies |

The pipeline throughput is limited by the slowest stage. Group 2, which processes 16 shuffle blocks across four spatial resolutions, dominates the pipeline latency. The ping_pong_sel flag alternates on each accepted image, directing Group 1 and the PS image writer to opposite banks of photo_mem.

## 3.4 Memory Architecture

Five categories of on-chip memory are used, each optimized for its access pattern:

| Buffer | Type | Size | Access Pattern | Purpose |
| --- | --- | --- | --- | --- |
| photo_mem | Ping-pong SDP BRAM (3 ch × 2 banks) | 3 × 50,176 × 8-bit | PS writes, G1 reads sequentially | Input image storage |
| maxpool_mem | SDP BRAM (24 channels) | 24 × 3,136 × 10-bit | G1 writes, G2 reads | G1→G2 feature map |
| DW buffer | Distributed RAM (combinational read) | 58 × 784 × 12-bit | G2 DW writes, G2 PW reads | Internal G2 intermediate |
| extra_mem | SDP BRAM (29 channels) | 29 × 1024 × 12-bit | G2 PW writes, G3 reads sequentially | G2→G3 feature map |
| Weight ROMs | Distributed/Block RAM (per layer) | See Section 4.5 | Read-only during inference | BN-folded weights and biases |

# 4. RTL Implementation

Figure 2 shows the internal dataflow architecture of the accelerator, illustrating the three processing groups and their interconnections through the on-chip memories.

Figure 2: Accelerator internal architecture and dataflow. Data flows top-to-bottom through Group 1 → maxpool_mem → Group 2 → extra_mem → Group 3.

## 4.1 Common Arithmetic Primitives

All convolution and fully connected computations are built from three fundamental hardware primitives that are shared across all processing groups. This modular approach ensures consistency in arithmetic behavior and simplifies verification.

### 4.1.1 MAC Unit (mac_unit.v)

The mac_unit implements a pipelined signed multiply-accumulate (MAC) operation. It is parameterized by the data width (DATA_W), weight width (W_W), the number of LSBs to drop after multiplication (DROP_LSB), and the output width (MUL_OUT_W). The operation for Group 1 (DATA_W=8, W_W=12, DROP_LSB=5, MUL_OUT_W=15) is:

product = signed(a_data) × signed(b_weight)   [8×12 = 20-bit signed product]

p_out   = product[DROP_LSB + MUL_OUT_W − 1 : DROP_LSB]   [bits 19:5 = 15 bits]

A pipeline register on p_out gives 1-cycle latency. The en input gates the register update (en=0 holds the output). The rst input asynchronously clears p_out to zero. The DROP_LSB parameter is chosen per layer so that the 8 fractional bits of the Q6.8 format are correctly aligned in the MAC output.

### 4.1.2 Adder Trees (adder_tree_N.v)

N-input adder trees are implemented using cascaded Adder3 carry-save adder (CSA) cells. The Adder3 module takes three N-bit inputs and produces one (N+2)-bit sum using a full-adder array for the CSA stage followed by a ripple-carry adder for the final combination. This is purely combinational — no pipeline registers. Each Adder3 level grows the output by exactly 2 bits, allowing precise tracking of bit growth through the tree.

Four tree sizes are implemented, each specialized for one layer type:

| Module | Fan-in | Adder3 Levels | Bit Growth | Layer Application |
| --- | --- | --- | --- | --- |
| adder_tree_9 | 9 | 2 | +4 bits | Group 1 conv3×3 (3 channels × 3 rows × 1 column) |
| adder_tree_12 | 12 | 3 | +6 bits | Group 2 PW 1×1 (12 parallel input channels) |
| adder_tree_29 | 29 | 4 | +8 bits | Group 3 PW 1×1 (29 input channels) |
| adder_tree_32 | 32 | 4 | +7 bits | Group 3 FC (32 parallel input channels) |

### 4.1.3 Saturating Quantizer (quantizer.v)

The saturating quantizer clips an extended-width accumulator output to the per-layer output width. It is parameterized by IN_W (input width), OUT_W (output width), and HAS_RELU (0 = two-sided, 1 = one-sided). The operation is:

if HAS_RELU=1 (one-sided / post-ReLU):

if input < 0:          output = 0

elif input > DATA_MAX: output = DATA_MAX  [DATA_MAX = 2^(OUT_W−1) − 1]

else:                  output = input[OUT_W−1:0]

if HAS_RELU=0 (two-sided):

if input > DATA_MAX:   output = DATA_MAX

elif input < DATA_MIN: output = DATA_MIN  [DATA_MIN = −2^(OUT_W−1)]

else:                  output = input[OUT_W−1:0]

The quantizer is purely combinational. HAS_RELU=1 is used for post-ReLU layers (Group 1 conv, Group 2 PW) where negative outputs are invalid. HAS_RELU=0 is used for DW convolution (no ReLU in depthwise) and the FC layer (negative class scores are meaningful).

## 4.2 Group 1 — 3×3 Convolution and Max Pooling

Group 1 implements the first stage of the network: a 3×3 strided convolution with 24 output filters on 3 input channels (RGB), followed by a 3×3 max pool with stride 2. The group transforms the 224×224×3 input image into a 56×56×24 feature map that feeds Group 2 through maxpool_mem.

### 4.2.1 Sliding Window FIFO (fifo_3x3.v, fifo_ctrl.v)

A 3×3 convolution requires simultaneous access to a 3×3 neighbourhood of pixels. For a streaming input (pixels arriving one per clock in raster scan order), this requires buffering 2 complete rows plus a partial third row. For a 224-pixel-wide image, this is 2×224 + 3 = 451 elements per channel, plus rounding to the nearest power of 2 for efficient implementation (depth = 455 = 2×226 + 3).

fifo_3x3 implements a depth-455 shift-register FIFO using SRL32 primitives. The output provides three row taps at the current column position: row 0 (current), row 1 (one row delayed), and row 2 (two rows delayed). Three FIFO instances per input channel (one per colour channel: R, G, B) provide the 9 pixel values needed for the 3×3 window simultaneously.

fifo_ctrl implements a 5-state FSM that sequences the FIFO loading and convolution operations: IDLE (waiting for start), LOAD_ROW (load initial rows with left-edge zero padding), LOAD_WIN (load the first 3-column window with left padding), PROCESS (generate valid output for each column), and LAST_PAD (flush the final windows with bottom-row zero padding). The FSM handles both stride-1 (same-resolution) and stride-2 (halving) configurations.

### 4.2.2 Convolution Core (conv3x3_core.v, conv3x3_filter_unit.v)

conv3x3_core instantiates 24 parallel conv3x3_filter_unit modules, one per output filter. All 24 share the same 9 pixel inputs from the sliding window FIFO, but each has its own 9×12-bit weight ROM output and 15-bit bias. The shared pixel path eliminates 23 redundant pixel memory reads per cycle, significantly reducing the BRAM read bandwidth requirement.

The datapath of a single conv3x3_filter_unit (Group 1 parameters) is:

| Stage | Operation | Width | Note |
| --- | --- | --- | --- |
| Input | 9 pixels × 3 channels = 9 taps | 8-bit each (PHOTO_W) | Q6.8 format, pixel values |
| Weights | 9 weights from ROM | 12-bit each (G1_CONV_WW) | BN-folded Q2.9 |
| MAC ×9 | signed(pixel) × signed(weight) >> 5 | 15-bit each | DROP_LSB=5 |
| adder_tree_9 | Sum 9 MAC outputs | 19-bit (15+4) | +4 from 2 Adder3 levels |
| Accumulator | Accumulate over 3 channels | 21-bit (19+2) | ceil(log2(3))=2 extra bits |
| Bias add | 21-bit acc + 15-bit bias | 22-bit | 2-input adder, +1 bit |
| Quantizer | Saturate to G1_FM_W | 10-bit (G1_FM_W) | HAS_RELU=1, one-sided |

The 3-cycle accumulation uses acc_clr to reset the accumulator: acc_clr=1 for the first two clock cycles (MAC pipeline filling — output is from previous/reset state), acc_clr=0 from the third cycle onward. The result is valid after 5 clock cycles per output pixel (3 drive + 1 wait + 1 check).

### 4.2.3 Max Pooling Core (maxpool_core.v)

maxpool_core implements a 3×3 max-of-9 pooling operation across all 24 channels simultaneously. Each channel uses a 3-stage comparator tree: Stage 1 computes 4 pairwise maximums (comparing taps 0-1, 2-3, 4-5, 6-7), Stage 2 computes 2 further maximums plus bypass of tap 8, and a pipeline register separates Stage 2 from Stage 3 (the final max-of-3 comparison). All 24 channels operate identically and in parallel — the pooling core introduces exactly 1 clock cycle of pipeline latency. Output width is G1_FM_W = 10 bits.

## 4.3 Group 2 — Depthwise and Pointwise Convolution

Group 2 is the most computationally intensive component, implementing 16 ShuffleNet V2 shuffle blocks organized across three spatial stages. The hardware must handle four distinct feature map sizes (56×56, 28×28, 14×14, 7×7) and two stride modes (stride-1 for most blocks, stride-2 for stage transitions), all using the same hardware with reconfigurable parameters.

### 4.3.1 Depthwise 3×3 Convolution (dw_conv3x3_core.v)

dw_conv3x3_core instantiates 58 parallel dw_conv3x3_filter_unit modules. In depthwise convolution, each filter operates independently on its own dedicated input channel — filter f reads only from channel f of the input feature map. This is fundamentally different from standard convolution where all filters share all input channels. The result is that all 58 output channels can be computed simultaneously with no data dependency between filters.

Each dw_conv3x3_filter_unit computes a single-pass (no multi-step accumulation) depthwise convolution:

| Stage | Operation | Width | Note |
| --- | --- | --- | --- |
| Input | 9 taps from group2_fifo | 10-bit each (G1_FM_W) | Post-maxpool feature map |
| Weights | 9 DW weights from g2_dw_weight_rom | 15-bit each (G2_DW_WW) | BN-folded |
| MAC ×9 | signed(feat) × signed(weight) >> 5 | 20-bit each | DROP_LSB=5 |
| adder_tree_9 | Sum 9 MAC outputs | 24-bit (20+4) | Combinational |
| Bias add | Add DW bias | 25-bit | Single addition |
| Quantizer | Saturate to G2_FM_W | 12-bit | HAS_RELU=0 — DW has no ReLU |

The 58 variable-width FIFOs (group2_fifo) support four width configurations controlled by width_sel[1:0]: W=7 (stride=9), W=14 (stride=16), W=28 (stride=30), W=56 (stride=58). The six distinct (W, H, STRIDE) configurations each get their own group2_fifo_ctrl FSM instance, with only the active configuration driving the shared FIFO through a multiplexer.

### 4.3.2 Internal DW Result Buffer

A key architectural decision in Group 2 is the internal DW result buffer. After the depthwise phase completes for an entire image pixel region, all 58 channel outputs must be available to the pointwise phase for multi-step accumulation. Rather than routing these through external ports, they are stored in a 58-instance × 784-word × 12-bit distributed RAM buffer inside group2_top.

The 784-word depth accommodates the maximum pixel count across all configurations (16 accumulation groups × 49 pixels = 784 for the 7×7 output). The buffer uses combinational (asynchronous) reads with synchronous writes, allowing the PW phase to access any channel result immediately without a read-latency pipeline stage. This eliminates the external dw_results_flat, dw_wr_addr, and dw_we ports that existed in the original design, simplifying the group2_top interface significantly.

### 4.3.3 Pointwise 1×1 Convolution (conv1x1_core.v)

conv1x1_core instantiates 58 parallel conv1x1_filter_unit modules. Each filter accumulates over 12 input channels per step (G2_PW_PAR_CHAN=12, reduced from 29 to fit within the ZU19EG DSP budget). For configurations requiring more than 12 channel accumulations, the controller (conv1x1_ctrl) sequences multiple steps — for example, the 58-channel configuration uses N_ACC=5 steps (5 × 12 = 60, covering all 58 channels with 2 zero-padded).

Each conv1x1_filter_unit datapath:

| Stage | Operation | Width | Note |
| --- | --- | --- | --- |
| Input | 12 channels from DW buffer (per step) | 12-bit each (G2_FM_W) | Step-loaded from buffer |
| Weights | 12 PW weights from g2_pw_weight_rom | 11-bit each (G2_PW_WW) | BN-folded |
| MAC ×12 | signed(feat) × signed(weight) >> 8 | 15-bit each | DROP_LSB=8 |
| adder_tree_12 | Sum 12 MAC outputs | 21-bit (15+6) | +6 from 3 Adder3 levels |
| Accumulator | Multi-step accumulation over N_ACC steps | 26-bit | 5-bit step counter |
| Bias add | Add PW bias (13-bit) | 27-bit | After accumulation complete |
| Quantizer | Saturate to G2_FM_W | 12-bit | HAS_RELU=1, one-sided |

conv1x1_ctrl generates acc_clr=1 for the first two accumulation steps (MAC pipeline settling period) and acc_clr=0 thereafter. The we (write enable) signal pulses on step N_ACC, triggering a write to extra_mem via accelerator_top. The 5-bit step_out signal (step_cnt+1, reflecting the current pipeline state) addresses the PW weight ROM, which uses registered (BRAM) reads with 1-cycle latency compensated by the +1 offset.

### 4.3.4 Group 2 Sequencer (group2_ctrl.v)

group2_ctrl implements a 4-state FSM (IDLE → S_S2 → S_S3 → S_S4) that orchestrates the sequential DW-then-PW execution across all 16 shuffle blocks. The controller maintains a 5-bit loop counter (0..15) and a phase flag (DW phase or PW phase). Done signals from the DW and PW controllers are latched in got_dw and got_1x1 flags, allowing the FSM to detect completion asynchronously. Stage transitions occur at loops=4 (S2→S3) and loops=12 (S3→S4), corresponding to the stride-2 blocks that halve the spatial resolution.

At each stage transition, the width and stride parameters change:

| Loops | Stage | Width | Stride | Output feature map |
| --- | --- | --- | --- | --- |
| 0 | S2 first block | 56 | 2 (stride-2) | 28×28 |
| 1-3 | S2 remaining | 28 | 1 | 28×28 |
| 4 | S3 first block | 28 | 2 | 14×14 |
| 5-11 | S3 remaining | 14 | 1 | 14×14 |
| 12 | S4 first block | 14 | 2 | 7×7 |
| 13-15 | S4 remaining | 7 | 1 | 7×7 |

## 4.4 Group 3 — Pointwise Conv, Average Pool, FC, and Argmax

Group 3 performs the final stages of classification: a 1×1 pointwise convolution expanding 464 channels to 1024, global average pooling over the 7×7 spatial extent, a 1024→1000 fully connected layer, and a running argmax to determine the top-1 class.

### 4.4.1 Group 3 Pointwise Convolution

conv1x1_g3_core uses 16 parallel filter groups (G3_PW_PAR_FILT=16), each processing 29 input channels (G3_PW_PAR_CHAN=29). A key distinction from Group 2 is that the output is full-precision 27-bit (G3_CONV_OUT_W=27) — no quantizer is applied. This preserves the full dynamic range of the accumulated sum for the subsequent average pooling operation, improving accuracy over a quantized intermediate.

The MAC per input: G2_FM_W(12) × G3_PW_WW(9), DROP_LSB=7, MAC_OUT=14 bits. adder_tree_29 sums 29 MAC outputs to 22 bits (+8). Multi-step accumulation over 64 filter groups × 17 steps each produces BIAS_OUT_W=27 bits.

### 4.4.2 Global Average Pooling

avg_pool_core accumulates all 49 pixel values (7×7 spatial) for each of 16 channels. The accumulator width is ACC_W = 27 + 6 = 33 bits (6 extra bits for log2(49)). After 49 clock cycles with acc_en=1, a result_we pulse captures the accumulated sum into a 9-bit output register via the one-sided quantizer. The quantizer effectively implements the 1/49 division through the Q6.8 fixed-point representation: the upper 9 significant bits of the 33-bit accumulator correspond to the averaged value in Q-format.

### 4.4.3 Fully Connected Layer

fc_core implements the 1024→1000 FC layer with 32-channel parallel processing (G3_FC_PAR_CHAN=32). Each of the 32 fc_filter_unit instances accumulates 32 input channels per step over 32 steps (1024/32=32 total steps). The fc_filter_unit datapath uses DROP_LSB=4 and includes a critical arithmetic right-shift by 2 before the quantizer (a scaling correction to prevent accumulator overflow with 32-step accumulation). The two-sided quantizer (HAS_RELU=0) is used since FC outputs can be negative (class scores are not constrained to be positive).

A running argmax unit updates the current maximum class score and its index as each of the 1000 classes is computed. After all 1000 FC outputs are produced, class_idx[9:0] contains the top-1 prediction.

### 4.4.4 Group 3 Sequencer (group3_ctrl.v)

group3_ctrl implements a 3-state FSM (IDLE → S_CONV → S_FC) with five internal counters: filter_group (0..63), pixel (0..48), conv_step, fc_step (0..31), and class_idx counter (0..999). A critical implementation detail is that extramem_rd_addr must be explicitly reset to 0 at each filter-group boundary — without this reset, all 64 filter groups after the first would read incorrect addresses from extra_mem.

## 4.5 Weight Storage and BN Folding

All layer weights and biases are stored in on-chip memory, organized for maximum throughput matching the parallelism of each layer. The BN folding script (scripts/group2/extract_weights_g2_g3.py) processes the pretrained torchvision ShuffleNet V2 × 1.0 checkpoint through three steps:

BN absorption: fold γ, β, μ, σ into conv weights using the formula in Section 2.2

Fixed-point quantization: round to the per-layer bit width, clip to representable range

Hex file generation: write packed binary representations as $readmemh-compatible .hex files

ROM organization is optimized for the access patterns of each layer:

| ROM Module | Type | Entries | Entry Width | Packing |
| --- | --- | --- | --- | --- |
| g2_dw_weight_rom | Distributed | 16 | 7,830 bits | All 58 DW weights per loop group |
| g2_dw_bias_rom | Distributed | 16 | 870 bits | All 58 DW biases per loop group |
| g2_pw_weight_rom | Block RAM | 320 | 7,656 bits | 58 × 12 weights per step |
| g2_pw_bias_rom | Distributed | 16 | 754 bits | All 58 PW biases per loop group |
| g3_pw_weight_rom | Block RAM | 1,024 | 4,176 bits | 16 × 29 weights per step |
| g3_pw_bias_rom | Distributed | 64 | 240 bits | All 16 PW biases per filter group |
| g3_fc_weight_rom | Block RAM | 32,000 | 288 bits | 32 FC weights per class/step |
| g3_fc_bias_rom | Distributed | 1,000 | 11 bits | 1 FC bias per output class |

The g2_pw_weight_rom uses Block RAM (rather than distributed RAM) to avoid excessive LUT usage — the 320-entry × 7,656-bit ROM would require approximately 38,000 LUTs as distributed RAM. The BRAM implementation uses registered reads with 1-cycle latency, compensated by conv1x1_ctrl outputting step_cnt+1 on the step_out bus to pre-fetch the next step's weights.

## 4.6 Split Synthesis Flow

Monolithic synthesis of the complete design requires approximately 34 GB of RAM for Vivado's out-of-context (OOC) synthesis of the shufflenet_board_top module, exceeding the 16 GB available on the development workstation. A custom split synthesis methodology was developed to overcome this:

Step 1 — Project Creation (build_project.tcl): The Vivado block design is created with black-box stub modules for Group 1, Group 2, and Group 3. Each stub declares the correct port interface with the (* black_box *) Verilog attribute, which prevents Vivado from attempting to synthesize the stub's empty body while preserving the module boundary in the project netlist. The OOC synthesis for the board top module is configured with flatten_hierarchy=none to ensure that these empty black-box boundaries survive the synthesis optimization passes (with flatten_hierarchy=rebuilt, empty modules have no primitives to rebuild and are silently removed from the netlist).

Steps 2–4 — OOC Group Synthesis (part0/1/2_synth_*_ooc.tcl): Each group is synthesized independently in Vivado non-project batch mode using synth_design -mode out_of_context -flatten_hierarchy rebuilt. The synthesis runs in a dedicated working directory, where the relevant hex weight files are pre-copied so that $readmemh can locate them using basename-only paths. The output is a design checkpoint (.dcp) file for each group.

Step 5 — Integration and Implementation (part3_synth_and_impl.tcl): The Vivado project is opened, synth_1 is run (synthesizing only the lightweight board wrapper and controller logic). After synthesis, the black-box cells are identified dynamically using get_cells -hierarchical -filter {REF_NAME =~ *groupN_top*} and filled with the pre-built OOC DCPs using read_checkpoint -cell. Implementation then proceeds with the fully merged design.

The cell reference names are prefixed by Vivado during OOC synthesis (e.g., group1_top → system_shufflenet_board_top_0_0_group1_top). The wildcard filter REF_NAME =~ *group1_top* reliably matches the renamed reference regardless of prefix.

# 5. Verification

## 5.1 Unit-Level Testbench Methodology

Every synthesizable RTL module has a dedicated self-contained, self-checking Verilog testbench. "Self-contained" means no external hex vector files are required — all test inputs and expected outputs are computed inline within the testbench Verilog using a golden model function. This design choice ensures testbenches are portable, easy to run, and cannot fail due to missing files.

Each testbench follows a consistent structure:

A golden model function implemented in Verilog that replicates the bit-true arithmetic of the DUT: signed multiplication, LSB dropping, carry-save addition, saturating quantization — all using the same parameterization as the RTL module under test.

A run_test task that applies stimulus at negedge clock edges (to avoid setup/hold races), waits for the pipeline to produce valid output, then compares the DUT output to the golden model expected value.

A pass_cnt/fail_cnt counter pair and a final RESULT line that confirms all test cases passed.

Key implementation details discovered during testbench development that are important for future debugging:

conv1x1_filter_unit (Group 2 PW accumulator): The accumulator register is gated by the en signal. When en=0 during the wait cycle, the accumulator holds its value — the effective accumulation count is exactly nacc.

fc_filter_unit (Group 3 FC accumulator): The accumulator is NOT gated by en. Even with en=0, the held MAC output continues to be added to the accumulator each clock cycle. The effective accumulation count is nacc+1. Additionally, an arithmetic right-shift by 2 (>>2) is applied inside fc_filter_unit before the quantizer.

avg_pool_core: The quantizer saturates at the signed half-range threshold 2^(DW−1)=256 (for OUT_W=9), not at the unsigned maximum 2^DW=512. Test values must keep N_PIX × pix_val < 256 for non-saturating tests.

maxpool_core: The pipeline register for bank selection (mem_select_r) adds 1 extra cycle of latency beyond the BRAM read latency, requiring 2 posedges after applying new inputs before the output is valid.

| Group | Modules | TBs Written | TBs Passing |
| --- | --- | --- | --- |
| Common | 9 | 9 | 9 ✓ |
| Group 1 | 10 | 10 | 10 ✓ |
| Group 2 | 14 | 14 | 14 ✓ |
| Group 3 | 11 | 11 | 11 ✓ |
| Memories + AXI | 2 | 2 | 2 ✓ |
| Top level | 2 | 2 | 2 ✓ |
| **Total** | **48** | **48** | **48 ✓ (100%)** |

## 5.2 Software Model Verification

The bit-accurate Q6.8 Python model (scripts/shufflenet_q68.py) implements the complete ShuffleNet V2 forward pass using Python integer arithmetic that mirrors every RTL operation: signed fixed-point multiplication with LSB truncation (using Python's integer right-shift), carry-save addition emulated through integer summation, saturating clipping, and BN-folded weight application. The model is loaded with pretrained weights from torchvision and quantized to Q6.8 using the folding script.

Verification results:

BN folding maximum coefficient error: 7.63×10⁻⁶ (well below 1 LSB = 1/256 ≈ 3.9×10⁻³ in Q6.8)

Top-1 accuracy on 100 ImageNet validation images: ~71%

Comparison with the floating-point torchvision baseline (~69.4% Top-1) shows the Q6.8 quantization introduces negligible accuracy loss for this network

A dummy 224×224×3 input image was used to verify end-to-end signal flow through the Python model, producing class index 499 consistently — confirming correct weight loading and forward pass execution.

# 6. HLS Reference Models

Four Vitis HLS C++ models implement the identical ShuffleNet V2 × 1.0 network targeting xczu19eg-ffvc1760-1-i. Each model builds on the previous, adding optimization directives that progressively trade implementation complexity for improved performance. The models share the same C++ testbench (hls/tb/tb_shufflenet.cpp) that exercises C-simulation and verifies functional correctness before synthesis.

## 6.1 Model 1 — Baseline (Sequential)

The baseline model (shufflenet_hls.cpp, solution "solution_baseline") implements a direct C++ translation of the ShuffleNet V2 forward pass with no HLS optimization pragmas. Loops execute one iteration per clock cycle with no pipelining or parallelism. This model establishes the reference latency — expected to be in the hundreds of millions of clock cycles for a full 224×224×3 image — and the minimum resource utilization baseline. Resource usage is expected to be dominated by BRAM for weight storage, with minimal DSP usage.

## 6.2 Model 2 — Pipelined

Model 2 adds #pragma HLS PIPELINE directives to the inner loops of each layer function, instructing Vitis HLS to schedule multiple loop iterations in a pipelined fashion. The target initiation interval (II) is 1 cycle per spatial position within each pipelined loop. Vitis HLS inserts pipeline registers between successive stages, increasing DSP and register usage but dramatically reducing per-frame latency.

Running Model 2: vitis_hls -f hls/scripts/create_hls_project.tcl

## 6.3 Model 3 — Parallelism

Model 3 adds full loop unrolling (#pragma HLS UNROLL) within the pipelined outer loops to expose maximum data-level parallelism. The 1×1 convolution functions are specialized per channel count (conv1x1_f58, conv1x1_f116, conv1x1_f232). Array partitioning (#pragma HLS ARRAY_PARTITION cyclic) enables parallel access to weight arrays matching the unroll factor.

Challenge encountered: The inner accumulation loops of conv1x1_f116 (116 input channels, fully unrolled) require the Vitis HLS scheduler to handle 116 × 116 = 13,456 simultaneous operations. On a 16 GB workstation, the HLS scheduler consumed all available memory (8000+ CPU-seconds) before completing. The same issue occurs for conv1x1_f232 (53,824 parallel paths).

Resolution: The UNROLL factor was reduced from full to factor=16 (116-ch) and factor=32 (232-ch). This reduces the scheduler's parallel path count to 116/16 × 116 = 8 × 116 = 928 and 232/32 × 232 ≈ 1,682 respectively — well within scheduler capacity while still demonstrating significant loop-level parallelism.

Running Model 3: vitis_hls -f hls/scripts/create_parallelism_project.tcl

## 6.4 Model 4 — Final (Combined Optimizations)

Model 4 combines all prior optimizations with three additional improvements:

Tighter clock target: 7 ns period (143 MHz) to guide Vivado's place-and-route toward a higher-frequency solution (the thesis target after export was 80 MHz after Vivado relaxation).

Dual-window DW convolution: two adjacent output columns are computed per iteration by sharing the 9-tap weight load, achieving approximately 1.67× throughput improvement over the single-window approach (6 cycles for 2 outputs versus 5 cycles each).

Dynamic quantization: per-layer bit widths are narrowed (10-bit for conv1 and maxpool, 12-bit for PW layers, 9-bit for DW and FC) to reduce resource usage at the cost of minor accuracy impact.

Running Model 4: vitis_hls -f hls/scripts/create_final_project.tcl

# 7. Design Optimizations

Eight optimizations were applied over the baseline 15-bit uniform datapath. Each optimization is described with its motivation and measured or expected benefit.

| # | Optimization | Description | Benefit |
| --- | --- | --- | --- |
| 1 | Per-layer FM widths | Feature maps reduced from 15-bit to 8/10/12/9-bit based on dynamic range analysis per layer | Reduced BRAM read width, LUT count, and routing congestion |
| 2 | Per-layer weight widths | Weights reduced from 15-bit to 9–15 bits. G1=12-bit, G2-DW=15-bit, G2-PW=11-bit, G3=9-bit | Reduced ROM size and DSP input width (DSP48E2 supports 18×27 signed) |
| 3 | 8-bit photo memory | photo_mem reduced from 15-bit Q6.8 storage to 8-bit pixel storage (PHOTO_W=8) | Halved image BRAM requirement; 8-bit pixels sufficient for visual accuracy |
| 4 | G3 quantizer removal | Group 3 PW conv1×1 output is kept as full 27-bit precision — no intermediate quantization | Preserves dynamic range for average pooling; avoids accuracy loss from premature clipping |
| 5 | Average pool approximation | 1/49 ≈ 5/256 applied through the Q6.8 format (no explicit multiply/divide needed) | Eliminates divider logic; averaging emerges naturally from the fixed-point format |
| 6 | FC pre-quantizer shift | Arithmetic right-shift by 2 inside fc_filter_unit before the saturating quantizer | Prevents 26-bit accumulator overflow during 32-step FC accumulation |
| 7 | Synchronous reset tree | High-fanout asynchronous resets replaced with a synchronous 4-stage reset tree from the MMCM output | Eliminated async reset timing violations; improved WNS by ~0.5 ns |
| 8 | Vivado directives | Synthesis: PerformanceOptimized strategy. Implementation: ExplorePostRoutePhysOpt (post-route physical opt) | Timing closure at 100 MHz; WNS improved from ~−0.3 ns to +0.125 ns |

The most impactful single optimization was the reduction of G2_PW_PAR_CHAN from 29 to 12, which reduced total DSP count from 2,916 (exceeding the ZU19EG's 1,968 available) to 1,930 (fitting with 38 DSPs of headroom). This required redesigning the conv1x1_filter_unit (12 MACs + adder_tree_12 instead of 29 MACs + adder_tree_29), updating conv1x1_ctrl to a 5-bit step counter (to support N_ACC up to 31), and reworking the DW buffer addressing scheme.

# 8. Implementation Results

## 8.1 Post-Implementation Resource Utilization

The following table shows the complete resource utilization report after place-and-route on xczu19eg-ffvc1760-1-i:

| Resource | Used | Available | Utilization (%) | Notes |
| --- | --- | --- | --- | --- |
| DSP48E2 | 1,550 | 1,968 | 78.76 | Reduced by P&R physical optimization |
| BRAM Tile | 550.5 | 984 | 55.95 | FC weight ROM dominates (~256 tiles) |
| LUT | 68,299 | 522,720 | 13.07 | Including LUT-RAM for DW buffer |
| Flip-Flop | 15,285 | 1,045,440 | 1.46 | Pipeline registers and FSM state |
| CARRY8 | 3,446 | 65,340 | 5.27 | Adder tree carry chains |
| URAM | 0 | 128 | 0.00 | Not used |

Note: The post-implementation DSP count (1,550) is lower than the design-time estimate (1,930) because Vivado's physical optimization merged some DSP multiplication chains during place-and-route. This is expected behavior and represents an additional resource saving.

## 8.2 Timing Closure

Full timing closure was achieved at the 100 MHz target after applying the PerformanceOptimized synthesis strategy and ExplorePostRoutePhysOpt implementation strategy:

| Metric | Value | Status |
| --- | --- | --- |
| Target period | 10.000 ns (100 MHz) | — |
| Worst Negative Slack (WNS) | +0.125 ns | ✓ CLOSED |
| Total Negative Slack (TNS) | 0.000 ns | ✓ No violations |
| Worst Hold Slack (WHS) | +0.011 ns | ✓ CLOSED |
| Worst Pulse Width Slack (WPWS) | +2.000 ns | ✓ CLOSED |
| Failing setup endpoints | 0 / 218,535 | ✓ PASS |
| Failing hold endpoints | 0 / 218,535 | ✓ PASS |
| Clock 1: clk_pl_0 (PS) | 100 MHz, auto-constrained | — |
| Clock 2: clk_100m (MMCM) | 100 MHz, auto-derived generated clock | — |

## 8.3 Verification Results

| Metric | Value |
| --- | --- |
| Total RTL modules | 50 (48 synthesizable + 2 stubs) |
| Testbenches written | 48 (one per synthesizable module) |
| Testbenches passing | 48 (100%) |
| Total test checks executed | >500 individual PASS/FAIL comparisons |
| RTL bug found by TBs | adder_tree_32 missing IN_W parameter in fc_filter_unit |
| Q6.8 model Top-1 accuracy | ~71% on 100 ImageNet validation images |
| BN-fold max coefficient error | 7.63×10⁻⁶ (< 1 LSB in Q6.8) |
| Board inference latency | Pending hardware deployment |
| Board throughput (fps) | Pending hardware deployment |
| On-chip power | Pending board measurement |

The adder_tree_32 IN_W parameter bug was discovered when fc_core testbench outputs were X (unknown) despite correct inputs. The root cause was that adder_tree_32 defaulted to IN_W=23 rather than the required MUL_W=14, causing a port width mismatch that propagated X through the adder tree. The fix was adding #(.IN_W(MUL_W)) to the instantiation.

# 9. Conclusion

This project delivered a complete, verified, and timing-closed RTL implementation of ShuffleNet V2 × 1.0 on the iWave ZU19EG Development Kit, alongside four Vitis HLS reference models for the same silicon. The RTL accelerator achieves 100 MHz operation with a worst-case timing slack of +0.125 ns, consuming 78.76% of DSPs, 55.95% of BRAM tiles, and 13.07% of LUTs on the ZU19EG.

The most technically challenging aspects of the project were:

Developing the split OOC synthesis methodology to overcome the 34 GB RAM requirement of monolithic synthesis on a 16 GB workstation.

Designing the pipelined accumulation controllers (conv1x1_ctrl, group3_ctrl) with correct timing for the MAC pipeline settling period and the registered BRAM read latency.

Reducing the Group 2 parallel channel count (G2_PW_PAR_CHAN: 29 → 12) to fit within the ZU19EG DSP budget while maintaining correct functionality.

Implementing the complete verification infrastructure (48 testbenches with inline golden models) and discovering the adder_tree_32 IN_W bug through testing.

## 9.1 Summary of Contributions

| Contribution | Detail |
| --- | --- |
| RTL design | Complete 50-module Verilog hierarchy, timing-closed at 100 MHz |
| Testbenches | 48 self-contained testbenches, all passing, 100% module coverage |
| Split synthesis | OOC flow reducing peak RAM from 34 GB to < 16 GB |
| Q6.8 model | Bit-accurate Python model, ~71% Top-1, BN-fold error < 8×10⁻⁶ |
| HLS models | 4 models with progressive optimization, same ZU19EG target |
| Bug discovery | adder_tree_32 IN_W mismatch in fc_filter_unit — caught by TB |

## 9.2 Future Work

Complete board-level inference: program ZU19EG with the generated bitstream, compile sw/shufflenet_test.c on the PS Linux image, and measure throughput, latency, and power.

Finalize the RTL vs. HLS comparison table with measured fps, energy-per-frame, and resource usage from all four HLS models after board-level execution.

Investigate quantization-aware training (QAT) using PyTorch to recover accuracy from the simplified DW→PW architecture and 8-bit pixel representation.

Scale the design to ShuffleNet V2 × 1.5 or × 2.0 by adjusting channel widths and verifying DSP budget compliance for larger configurations.

Explore partial reconfiguration to enable runtime switching between multiple network configurations using the Zynq UltraScale+ PR capability.

# References

[1] N. Ma, X. Zhang, H.-T. Zheng, and J. Sun, "ShuffleNet V2: Practical Guidelines for Efficient CNN Architecture Design," in Proc. ECCV, 2018.

[2] Xilinx Inc., "Zynq UltraScale+ MPSoC Data Sheet: Overview (DS891)," 2023.

[3] Xilinx Inc., "Vivado Design Suite User Guide: Synthesis (UG901)," 2024.

[4] Xilinx Inc., "Vitis High-Level Synthesis User Guide (UG1399)," 2024.

[5] iWave Systems, "iW-RainboW-G35M-19EG ZU19EG SOM Hardware User Manual," 2022.

[6] B. Jacob et al., "Quantization and Training of Neural Networks for Efficient Integer-Arithmetic-Only Inference," in Proc. CVPR, 2018.

[7] E. Gschwend, "ZynqNet: An FPGA-Accelerated Embedded Convolutional Neural Network," ETH Zurich, Master Thesis, 2016.

[8] C. Guo et al., "Angel-Eye: A Complete Design Flow for Mapping CNN onto Embedded FPGA," in IEEE Trans. CAD, 2018.